# Control Improvement Progress Report V3

## sEMG + IMU fusion, and the final benchmark

Date: 17 September 2026 (revised — the fusion is built, benchmarked and shipped)

This is an internal status note, not a polished deliverable. It summarises where the "smarter tremor control" work stands: what's built, what's still open, and what tripped us up. Kept light on the maths on purpose — the technical detail lives in docs/SEMG_INTENT_FUSION.md (new, and the main reference for this revision), docs/CONTROL_IMPROVEMENT_PLAN.md, docs/SEMG_IMPLEMENTATION.md and docs/SEMG_DATASET.md.

What changed in this revision. V2 ended with sEMG validated as a signal source but not connected to the control loop at all, and with the EKF not recommended as a replacement on the evidence then available. Both of those have changed. sEMG is now fused into the estimator, the combined system is the best arm on the benchmark by every measure we track, and the fusion works through a mechanism V2 did not anticipate. Sections 3–6 are new. Section 2 closes out V2's pending list item by item. Section 7 is an honest account of a new failure we introduced, found, partly fixed, and have not fully solved.

One-line summary. The corrected IMU-only estimator averaged 92.1% cancellation on the scenarios the hardware can actually serve, and met the 90% target on 11 of 16. Adding sEMG takes that to 93.0% and 15 of 16, while keeping voluntary-motion error at 0.207° (against 0.458° for the original controller) — and we can show by direct ablation that the sensor, not the mechanism, is what buys it.

# 1. The problem, restated

Two things had to be true at once for this project to succeed, and they pull against each other:

- Cancel the tremor. Hold the spoon level against 4–20 Hz involuntary motion.

- Don't cancel the person. When the user deliberately moves their hand, the spoon must follow, not fight.

The original controller (simple_pid) does (1) well and (2) badly: it averages 90.2% on the feasible scenarios but collapses to 51.5% when deliberate movement and tremor happen together, and it carries 0.458° of error on a movement-only scenario.

The EKF was introduced to do both, by estimating the voluntary and tremor parts of the motion separately rather than filtering a fixed frequency band. V2 showed it fixed (2) convincingly — the movement-only error dropped to 0.201° — but that it lost ground on (1) in a specific way, and V2's Section 6 traced why.

# 2. Closing out V2's pending list

V2 Section 8 listed seven open items. Where each now stands:

| V2 pending item | Status |
| --- | --- |
| Decide whether the EKF replaces the current controller | Resolved — yes, with sEMG. Section 5 |
| Decide how to score scenarios outside the hardware envelope | Resolved and implemented. Section 4 |
| Fix the inoperative joint-travel check in the authority limiter | STILL OPEN — not attempted this revision. Section 8 |
| Decide what to do about the single-frequency limitation | Resolved. The answer is not more oscillators; it is a second sensor. Section 3 |
| sEMG validated but not fused into the control loop | Resolved. Sections 3 and 5 |
| Three of six real-data sEMG checks unanswered | STILL OPEN — unchanged. Section 8 |
| Final side-by-side comparison report | Done. output/results/CONTROL_RESULTS.md, figures, and this note |

Five of seven closed. The two that remain are the two that need something we did not have this revision: a small code repair plus a measurement campaign, and a real recording session.

# 3. The main result: what sEMG is actually for

V2 Section 6 found the estimator's structural weakness. The tremor model carries one frequency. When a dominant tremor component claims it, weaker components at other frequencies have nowhere to live — and the "voluntary movement" part of the model, which is deliberately flexible, absorbs them instead. Absorbed tremor is then passed through to the spoon as though the user had meant it. Measured open-loop, an 8 Hz dominant tremor with a 3 Hz companion had 87.8% of that companion mis-labelled as voluntary movement.

The insidious part, and the reason this went unnoticed for so long: the model's own health signal looks perfect while this is happening. The two parts still add up to the measured motion, so the mismatch statistic stays tiny. The estimator is confidently wrong. No amount of watching the IMU can detect it, because the IMU only ever sees the sum.

## 3.1 The experiment that settled the design

Before touching sEMG we ran a deliberately boring experiment (run_intent_authority_experiment, no muscle signal involved anywhere). It turns one dial — how much freedom the estimator grants the "voluntary movement" part — and measures what happens.

| Scenario | dial ×1 | ×1/2 | ×1/8 | ×1/64 | ×1/512 |
| --- | --- | --- | --- | --- | --- |
| messy tremor (mixed) | 87.99% | 89.41% | 91.46% | 93.00% | 93.55% |
| messy tremor (irregular) | 87.81% | 89.26% | 91.32% | 92.85% | 93.39% |
| movement + tremor (combined) | 85.88% | 75.86% | 26.86% | −81.41% | −398.51% |
| movement only (intent) | 0.201° | 0.689° | 1.430° | 3.234° | 5.607° |

Read the bottom two rows against the top two. The setting that best suppresses messy tremor is the setting that destroys voluntary movement. At merely half the nominal value, the messy-tremor cases have gained 1.4 points while the movement case has already lost 10 and the movement-only error has more than trebled.

So there is no correct fixed value. The dial has to be turned differently at different moments, and something has to say which moment it is. That information does not exist in the IMU signal. It exists in the muscle.

This is the argument for sEMG, and it is a much better argument than the one we started with. We are not using sEMG because it is a nice extra sensor; we are using it because we can demonstrate a specific decision the system must make and cannot make without it.

## 3.2 What we fuse, and what we deliberately do not

The tempting design is to have the muscle signal estimate the voluntary hand angle and feed that into the model. We built that first and rejected it on measurement: the estimate correlates well with the truth (r = 0.95–0.98) but carries 1.6–1.8° of error, while the effect we are trying to remove is 0.02–0.18°. It would have injected an order of magnitude more error than it removed, directly into the reference the spoon tracks.

What we fuse instead is much weaker and much more reliable: is the muscle driving, and how hard? The difference between the two opposing muscles of a pair, smoothed, separates "tremor only" from "deliberate movement happening" by 6.3 : 1 across all twenty benchmark scenarios. That confidence turns the dial from Section 3.1, per axis, every sample.

Nothing else changes. The estimator keeps the same states, the same measurement model, and the same IMU-only view of attitude. sEMG has exactly one job, and it is the job the evidence supports it doing.

# 4. The hardware-envelope scoring decision (V2 §5.4, now settled)

V2 argued that scenarios which drive the mechanism past its physical limits should not be scored as control failures, but left the definition as a pending decision. It is now decided and implemented in code, so it can be audited rather than applied by hand.

The test is an inverse-dynamics feasibility bound (rcflex_case_feasible): assume perfect knowledge and exact tracking, and ask what current, voltage, copper power, travel and side load the input would demand. If the hardware cannot supply them, no controller could have succeeded, and scoring one measures the actuator rather than the algorithm. The same function now also generates the published envelope table, so the two cannot disagree.

Three of the twenty scenarios fail it:

| Case | Input | Why it fails |
| --- | --- | --- |
| 5 | 10 Hz, 3° | needs 7.10 V of a 7.4 V bus (and the bus sags under load) |
| 14 | 8 Hz, 5° | needs 7.50 V |
| 20 | 20 Hz, 1° | needs 1.84 A of a 1.5 A limit, 9.75 V, and 5.28 W against a 4.4 W continuous rating |

Case 5 is a new finding — V2 had only identified 14 and 20.

Three averages are now always reported together, so the exclusion is visible rather than silent: all scored scenarios, feasible scenarios only (the headline), and feasible scenarios that also stayed inside the limits. Quoting only the flattering one is precisely the failure this arrangement exists to prevent.

# 5. The final benchmark

Twenty scenarios, six controller variants, identical cases and identical sensor seeds, so the only difference between the blocks is the variant definition.

| Variant | What it is | All scored | Feasible | Worst feasible | Movement-only error | Meets 90% |
| --- | --- | --- | --- | --- | --- | --- |
| simple_pid | the original controller | 64.19% | 90.23% | 51.51% | 0.4583° | 15/16 |
| ekf_cv_legacy | first EKF, with the V2 bug | 68.75% | 88.96% | 30.75% | 1.2390° | 11/16 |
| ekf_cv | corrected EKF, IMU only | 71.03% | 92.12% | 85.88% | 0.2008° | 11/16 |
| ekf_cv_osc2 | + a second tremor oscillator | 71.04% | 92.13% | 85.90% | 0.2008° | 11/16 |
| ekf_cv_qfix | ablation — same mechanism, no sEMG | 65.62% | 85.18% | −41.41% | 3.0418° | 15/16 |
| ekf_cv_semg | sEMG + IMU (shipped) | 72.63% | 93.04% | 85.31% | 0.2070° | 15/16 |

Scenario by scenario, the combined system recovers exactly what the IMU-only EKF had lost:

| Scenario | original | IMU-only EKF | sEMG + IMU |
| --- | --- | --- | --- |
| 6 messy tremor (mixed) | 92.35% | 87.99% | 92.62% |
| 7 sweeping tremor (chirp) | 90.87% | 88.57% | 90.59% |
| 8 irregular tremor | 92.46% | 87.81% | 92.82% |
| 9 movement + tremor | 51.51% | 85.88% | 85.31% |
| 10 movement only (error) | 0.4583° | 0.2008° | 0.2070° |

This is the first variant that is not a trade-off. It matches the original controller on plain tremor, matches the EKF on deliberate movement, and is better than both on messy tremor.

## 5.1 How we know it is the sensor doing the work

This is the part that matters for defending the result, and it is why the ekf_cv_qfix ablation exists. That variant applies the identical dial-turning mechanism with the confidence held at a constant — the classic "just turn the dial down" fix — with no muscle signal at all. Against it:

|  | ablation (no sEMG) | sEMG + IMU | attributable to the sensor |
| --- | --- | --- | --- |
| Feasible-case average | 85.18% | 93.04% | +7.86 points |
| Movement + tremor (case 9) | −41.41% | 85.31% | +126.7 points |
| Movement-only error | 3.0418° | 0.2070° | 14.7× lower |

The ablation behaves exactly as Section 3.1 predicts: it wins on messy tremor and destroys everything involving deliberate movement. The sensor is what lets the system have both.

Looking inside the estimator directly, tremor mis-labelled as voluntary movement falls by 2.6–3.1× on the messy-tremor scenarios (0.171° → 0.066°, 0.170° → 0.055°, 0.038° → 0.014°), with no loss of movement tracking. That is the designed mechanism, observed working.

## 5.2 It is not paid for with actuator effort

Peak current across the whole sweep is 1.4305 A (IMU-only) against 1.4320 A (sEMG+IMU), and peak average coil power 5.7228 W against 5.7190 W. Worth stating explicitly: a cancellation gain bought with extra current would not survive the thermal and battery limits, and this one is not.

# 6. What the evidence does not say

The muscle signal is synthetic. It is generated from the same ground truth that drives the simulated hand, so it carries the information we are asking it for by construction.

Every number above is therefore an upper bound, and the result is a conditional: if a real recording separates the two situations as well as this one does, then the estimator improves by this much. It is an algorithmic result — the architecture is sound and the control benefit is real in simulation — and it is explicitly not evidence about human physiology. docs/SEMG_INTENT_FUSION.md §8 states the falsifiable targets a real recording would have to hit, chief among them a class separation of at least 4 : 1. We measure 6.3 : 1 here, which sounds comfortable but is not: the acceptance test's own worst-case margin is about 7%.

The real-data validation is deliberately kept as a separate stage with its own gate, and that gate is still open (Section 8).

# 7. A failure we introduced, found, and only partly fixed

This is the most important section for anyone judging the quality of the work, so it is written plainly.

The first version of the gate made deliberate movement worse, badly. The 20-scenario benchmark contains no fast voluntary movement — every movement in it is a slow, 5-second oscillation — so we added one (reach: a 0.8-second reach superimposed on tremor) precisely to test the thing we were most worried about. It failed:

|  | IMU-only | first gate | two-speed gate (shipped) |
| --- | --- | --- | --- |
| reach, mean error over 4 sensor seeds | 0.285° | 0.99° | 0.67° |

About 3.6× worse than doing nothing clever, and inconsistent between seeds. The cause is unglamorous: rejecting muscle activity caused by tremor needs heavy smoothing, heavy smoothing costs time, and the gate needed 0.3–0.5 s to notice a movement lasting 0.8 s.

The fix that didn't work, and why it's still interesting. The estimator already has a safety layer meant to restore flexibility during exactly this kind of transient, and our gate was overriding it. We rebuilt it so the gate could not override it — and got results identical to four significant figures. The layer never fires during a reach at all, because (as in Section 3) the model still matches the sensor beautifully while being wrong. This is the second independent confirmation of V2 §5.3's first bullet: that confidence signal is the wrong kind of alarm for this class of problem.

The fix that did work, partly. The gate now runs two speeds: a slow, clean path that alone may close it, and a fast path that may only ever open it. The asymmetry is the point — wrongly closing the gate suppresses the user's movement, whereas wrongly opening it merely reverts to the previous behaviour, so the two mistakes should not be treated equally. That recovered a third of the loss for about 0.3 points of tremor suppression.

It is not solved. The shipped system is still ~2.4× worse than the IMU-only estimator on a fast deliberate movement. We set a 350 ms response target in advance; the system achieves 404 ms. A 16-point sweep over the filter settings found nothing meeting both that target and the separation requirement — so we adopted none of them, rather than moving the target to whatever the design happened to achieve. The headline numbers in Section 5 do not reflect this cost, because the benchmark contains no such scenario. Anyone quoting the headline should quote this alongside it.

One more correction. At one point this revision recorded that the two versions of the safety-layer fix differed by 20 points. That comparison had been run at two different sensor seeds and was simply wrong; re-run properly, they are identical. The erroneous claim is struck through in the code comment and the design document rather than deleted, because a retracted measurement is itself a useful warning.

# 8. What's still open

- The joint-travel protection in the authority limiter is still inoperative. V2 diagnosed it: the value being checked is clipped to the limit before being compared against that same limit, so the check can never trigger. It was not attempted this revision. It is a small ordering fix, but whether repairing it actually helps the worst case is a separate question that needs measuring, not assuming.

- Fast deliberate movement (Section 7). Real, measured, and the largest remaining weakness of the delivered system.

- Three of the six real-data sEMG checks remain unanswered, unchanged from V2, because no public dataset records forearm muscle signals simultaneously with a physical tremor measurement. This now also gates the one number the whole fusion depends on — the 6.3 : 1 separation — which has never been measured on a human.

- A sustained isometric hold would read as deliberate movement. Holding a loaded spoon still, against gravity, produces muscle drive without motion; the gate would stay open and the system would revert to its IMU-only behaviour. That is the safe direction of failure, but it is a genuine gap and it is untested.

- The sEMG stream is still not wired into the Simulink model, only the MATLAB controller. Deferred deliberately; it needs a .slx port change.

- No hysteresis, dead zone or creep in the plant model — the project's standing caveat. This remains an algorithm-development baseline, not a validated digital twin.

# 9. Suggested next steps, in the order we would do them

- Repair the joint-travel check (one ordering fix), then re-run the two outside-envelope scenarios and measure whether it changes anything. Cheap, and it closes the oldest outstanding item.

- Attack the fast-movement gap properly. The current gate detects movement from muscle amplitude, which by definition arrives late. The more promising quantity is the rate of change of muscle drive, which leads amplitude substantially — we measured it as ~4× earlier detection, but it also amplifies noise, so it needs its own threshold derived the same way the current ones were. This is a contained experiment, not a redesign.

- Record a short synchronised session — forearm muscle sensors and a motion sensor worn together, alternating rest, tremor-like shaking and deliberate reaching. Twenty minutes of recording would close the three outstanding validation checks and produce the first real measurement of the class separation the whole fusion rests on. This is the single highest value item on the list and the only one that changes what the result means.

- Re-measure the calibration's stability across that session (fit the gain at the start and the end and check they agree), since a per-subject calibration that drifts within one meal would be a problem.

- Test an isometric hold explicitly, since it is the one everyday situation we know the design handles by giving up.

- Revisit whether case 5 changes the hardware sizing. Three of twenty scenarios now fail the feasibility bound on bus voltage or current. If those amplitudes are clinically realistic, the actuator or the battery is undersized, and that is a mechanical conclusion this control work has produced as a by-product.

Items 1, 2, 5 and 6 are all doable in simulation. Item 3 is the one that needs hardware and a person, and it is the one that would turn this from a defensible simulation result into a defensible system.

## Appendix: where the numbers come from

Every figure in this note is reproducible from the repository:

START_PROJECT('setup')

calibrate_rcflex_semg_intent()        % the gate constants and the 6.3:1 separation

run_intent_authority_experiment()     % Section 3.1, no sEMG involved

benchmark_rcflex_control()            % all six variants, 20 scenarios, fixed seeds

summarise_rcflex_benchmark()          % Section 5's three averages

plot_rcflex_benchmark('final')        % and ('fusion') to include the ablation

run_semg_intent_experiment()          % Section 5.1 and the 'reach' scenario

plot_rcflex_semg_gate('reach',8,1,4)  % Section 7 in time

run_validation()                      % 15 of 15 groups pass

Results land in output/results/ (control_benchmark.csv, control_benchmark_summary.csv, intent_authority_ceiling.csv, semg_intent_e2.csv, semg_intent_calibration.csv) and figures in output/figures/.

Figures worth putting in the written report: semg_intent_separation.png is the key one — it shows tremor absorbed and voluntary movement suppressed side by side, and the ablation wins one panel while losing the other, which is the whole argument in one image. bench_suppression.png is the headline comparison, intent_authority_ceiling.png shows why a fixed setting cannot work, and semg_gate_reach.png shows the mechanism operating in time including its remaining weakness.

With the sEMG switch off, the system reproduces every pre-fusion number exactly (the movement+tremor case is unchanged at 85.8826099585925%), so the comparison is a controlled one rather than an incidental one.